Using WAV Files With WriteLog

From Ron, N6EE

1. Writelog allows arbitrary sound file names to be inserted into the function key macro definition.  For example, you could name a CQ WAV file "SSCQ.WAV" and put that into the F2 key:

F2: <SSCQ.WAV>

2. This from the Writelog Help file:

Recording a new message on-the-fly

WriteLog enables you to record messages quickly. Type SHIFT+F2 to start the recording of a new message for F2 (or SHIFT+F3 for the F3 message, etc), and, after you have finished the message, type ESCAPE to save the recording. Typing SHIFT+F2 instead of ESCAPE cancels the recording and leaves the old message unchanged. Message recording is automatically cancelled after 60 seconds if you fail to type ESCAPE.

The names of the files created by the SHIFT function keys are:

MSG2.WAV will be recorded when you press SHIFT+F2.

MSG3.WAV will be recorded when you press SHIFT+F3....

MSG11.WAV will be recorded when you press SHIFT+F11.

3. Writelog allows the sound (WAV) files to be placed in any folder on your computer.  Use Setup-->Ports-->WAV File Location.

4. You can avoid the hassle of creating WAV files altogether if you have a digital voice recorder in your  CAT controlled transceiver.  For example, if you have a KDVR3 installed in the Elecraft K3, you can send the voice messages if you:

a. in Writelog.ini create a section labeled  [Elecraft_K3_Commands]

Macro_10=SWT21;    \\to send M1

Macro_11=SWT31;    \\to send M2

Macro_12=SWT35;    \\to send M3

Macro_13=SWT39;    \\to send M4 then,

b. define the function key macro using %GKn to send Macro_n, like this:

F2: %GK10

You can terminate a message immediately by tapping the footswitch or triggering Vox.

Using WAV Files With WriteLog

From Rebar, N6DB

“For proper audio levels, plug your headset mic into the radio (Heil adapter) and set

the mic gain for proper ALC (to end of red bar on ALC meter) with Processor OFF.

Then turn processor ON and adjust Processor control to get 4 or 5 db on

the COMP meter scale. Then leave Mic and Proc controls on radio alone.

Adjust computer message playback with the Writelog "Sound board mixer control"

you can access through the Start, Programs, Writelog folder.  On the Left, the

"Output Level" is the master to get playback up to the same ALC/COMP readings

as your microphone.  You can equalize relative balance with the middle controls

"Msg play level" versus the "Mic level". I don't like running Mic through the computer,

the quality is less than straight into the rig. But going through the computer allows record

on the fly if you need it.”   (Mark Aaker, K6UFO)

In Audacity, we have found that recording the messages with the following settings seems to work well:

-  Project Rate (Hz) = 11025 [lower left corner of application window]  Set this before making any recordings

-  Record message and add compression (default setting is OK)  'Effect-->Compressor...'

-  Remove Truncate silence (again default OK) 'Effect-->Truncate Silence...'

-  Export audio as windows WAV file and name msg#.wav where the msg# corresponds to the F-key.  For F1/F11 CQ, name file msg11.wav.

--